Report of Pesticide Residue  Monitoring Results

of the Czech Republic for 2007

Concerning Directive 90/642/EEC, 86/362/EEC

and Recommendation 2007/225/EC

August 2007

1. Summary

2. Organisation of the monitoring programmes and sampling

3. Quality assurance

4. Other information

Tables according to EU reporting format:

Table A1 – Part I 	Summary of numbers of samples, sample origins and result,  surveillance   sampling, national and co-ordinated programme

Table A1 – Part II	Summary of numbers of samples, sample origins and result, follow-up enforcement sampling, national and co-ordinated programme

Table A1 – Organic	Summary of numbers of samples and results

Table A2 – Part I 	Fruits and vegetables: Summary table of pesticides sought and found,     surveillance sampling, national and co-ordinated programme

Table A2 – Part II 	Cereals: Summary table of pesticides sought and found, surveillance sampling, national and co-ordinated programme

Table B:       	Notifications of the co-ordinated programme (specific exercise) to the         European Commission

Table C:	Notification of the results of Check sampling of the National programme to           the European Commission

Table D1:	Details of residues exceeding EC-MRLs, surveillance sampling of fruits,    vegetables and cereals, national and co-ordinated programme

Table D2:	Details of residues exceeding non-harmonised MRLs including national         MRLs,  surveillance sampling of fruits, vegetables and cereals, national and                       co-ordinated programme

Table D3:	Details of residues exceeding EC-MRLs , follow-up enforcement sampling of fruits, vegetables and cereals, national and co-ordinated programme

Table D4:	Details of residues exceeding non-harmonised MRLs including national         MRLs, follow-up enforcement sampling of  fruits, vegetables and cereals,     national and co-ordinated programme

Table E: 	Details of samples with multiple residues (>=2) in single samples surveillance     and follow-up enforcement sampling of fruits, vegetables and cereals, national     and co-ordinated programme

Table G:	Laboratory: Information about laboratories involved in the monitoring           exercise

1. Summary

In 2007, totally 840 samples of fruits, vegetables, cereals and processed products and 45 baby food products of domestic and non-domestic origin were analysed in the national and co-ordinated monitoring program. National or EU harmonised MRLs were exceeded in 30 samples (3,4 %). Pesticide residues above the reporting limits were found in 44 % of the surveillance products of plant origin. Regarding products of plant origin, domestic production made up 22,7 % of the samples, 57,9 % of the samples came from other EU countries and 19,4 % were from the third countries.  Apples, head cabage, leek, letucce, tomatoes, peaches (incl. Nectarines), rye, strawberries – totally 220 samples, were analysed in the of EU co-ordinated programme for the year 2007.  The level exceeding MRL was found out in 27 samples of fresh fruits and vegetables,  in 2 sample of baby food product and in 1 sample of meal.  Most of violative samples were from third countries (13), 11 samples were from EU countries and 6 samples from the Czech Republic.

64 different pesticide residues out of 184 sought analytes (including metabolites) in fruit and vegetables were found at least once during the monitoring programme. The most frequently found pesticide residues, in decreasing order of frequency (found/sought) are: orthophenylphenol, bromide ion, dithiocarbamates, imazalil, azoxystrobin, thiabendazole, chlormequat. Out of 182 pesticide residues (including metabolites)  sought for,  pirimiphos-methyl, chlormequat, deltamethrin was detected in cereals.

2. Organisation of Monitoring programmes and Sampling

Responsibilities

Czech Agriculture and Food Inspection Authority (CAFIA) is the competent authority for controls of pesticide residues in foodstuffs and provides the national and EU co-ordinated monitoring programmes in co-operation with the Ministry of Agriculture.

Design of Programmes

- The sampling plan for pesticide residues monitoring is always drawn up for one calendar year. The plan is elaborated  by the Headquarters of CAFIA as internal provision and it is distributed to the CAFIA regional inspectorates which are responsible for its implementation. The plan is based on Commission Recommendation concerning a coordinated Community monitoring programme which sets the minimum number of samples for the Czech Republic. Within the monitoring of pesticide residues, the Czech Republic analyses approximately 800 samples per year. Within pesticide residue monitoring, especially commodities mainly presented in the consumer basket are collected. In selection of commodities for the monitoring a number of findings in the past (in the Czech Republic and other member states) is also taken into account. Fresh fruit and vegetables and baby food comprise the primary proportion of samples taken.

Sampling

Sampling is performed in accordance with sampling procedures referred to in Commission Directive No. 2002/63/EC that has been incorporated into the national regulation for sampling for determination of pesticides in and on fruit and vegetables.

- Samples are taken by authorised and for this purpose trained employees of the CAFIA, i.e. inspectors of the CAFIA regional inspectorates. The main sampling points are importers, whosalers and retail shops for domestic and non-domestic products.

Enforcement

- A ban on sale/distribution of inspected foodstuffs is imposed when exceeding the MRL or it is ordered to withdraw unsatisfactory foodstuffs from circulation. Furthermore, the person inspected is ordered to take such measures that minimise the risk of further occurrence of limit-exceeding findings. A fine is imposed on the inspected person within administrative procedure. When determining its amount the importance, manners, duration and consequences of illegal act are taken into consideration.

3. Quality assurance

Accreditation

All analyses were carried out in the laboratory of Czech Agriculture and Food Inspection Authority (CAFIA) in Prague. Laboratory is accredited by Czech Accreditation Institute (CAI) according to the ISO/IEC 17025 standard for all methods used for monitoring and/or enforcement analysis.

Analytical methods

Pesticide residues reported by the laboratory in the year 2007 have been analysed by 5 analytical methods which covered together  184 analytes (incl. metabolites) – 2 multi-residue methods (first  MRM have been based on ethyl acetate extraction, GPC clean-up and capillary gas chromatography (GC-ECD/NPD/MSD), second MRM have been based on acetonitrile extraction followed by LC-MS/MS - liquid chromatography coupled with triple quadrupole) and 3 single-methods (GC-MSD for dithiocarbamates, GC-ECD for inorganic bromine and LC-MS/MS for chlormequate).

Proficiency testing

During the year 2007 the laboratory took part in 4 proficiency tests focused on pesticide residues – in three EUPT rounds (EUPT FV 09, EUPT FV LC1 and EUPT AO 02) and in one national test organised by CAFIA for laboratories in Czech Republic.

EU Quality Control procedures

Although most of requirements from the EU quality control guidelines (Document N° SANCO/10232/2006) have been fully implemented some of them are still fulfilled only partly. Improvement of QC procedures has been achieved since the year 2006 and the process will continue also in the future.

Uncertainty

A concentration-dependent uncertainty concept based on the Horwitz equation was applied for uncertainty estimation in the year 2007. For calculation of expanded uncertainty (U), the RSDR obtained from the equation was multiplied by a factor of 1.4, which was derived from laboratory experiments, validation and QC data. Since the end of the year 2007 the harmonised “50% uncertainty” approach recommended in the SANCO guidelines has been applied.

4. OTHER INFORMATION

Risk assessment

Health risk assessment is in the Czech Republic performed by the Ministry of Health. Non-complying samples are ceded to the MH and if they are relevant for transmission via RASFF (Rapid Alert System for Food and Feed), they are notified. In 2007, we notifed 2 findings of pesticide residues but as it concerned the baby food, those samples were not not ceded to the MH for risk assessment because there was no need to assess the risk. 

2007.0475 - CYPRODINIL,FLUDIOXONIL AND FENHEXAMID IN FRUIT PUREE WITH STRAWBERRIES FOR INFANTS FROM THE CZECH REPUBLIC 
2007.0782 - FENHEXAMID IN FRUIT PUREE WITH STRAWBERRIES FOR INFANTS FROM THE CZECH REPUBLIC, RAW MATERIAL FROM SPAIN AND THE CZECH REPUBLIC 

There was one case of pesticide residues notified in 2007 as well (2007.ACW -- Procymidone in fresh spinach from France) but the sample was already taken in 2006.